Specification Sheet – Tank

Project Code: 1403/15651
Client Name: MR. jan

1. Body & Material Specifications

- Material: Stainless Steel 316

- Inner shell diameter: 1900 mm

- Inner shell height: 1700 mm

- Inner shell thickness: 4 mm

- Outer shell thickness: 3 mm

- Bottom dish thickness: 5 mm

- Thermal insulation: Rock wool, 50 mm thickness

2. Agitation System

- Agitator type: Central paddle mixer

- Motor power: 1.5 kW

- Rotation speed: 20–30 RPM

3. Nozzles & Accessories

- Manhole: 400 mm

- Vent (air breather)

- Internal lighting system

- Sampling valve

- Rotary fittings: 38 mm (2 units)

- Discharge pipe: 63.1 mm

- Inlet pipe: 63.1 mm

4. Installation Heights

- Product outlet height from ground: 700 mm

5. Instrumentation

- Thermometer range: 0–120°C

6. Structural & Safety Elements

- Ladder with safety cage (access ladder with protective guard)

- Protective handrail system (platform safety railing)